1) Write down the energy transfers that take place in:

PD2 ENERGY REVISION QUESTIONS

- Starting a car

- The human stomach

- A coal furnace

- Turning the pedals of a bicycle

A hydro-electric power station (give ALL energy changes)

2) Copy and label this diagram of a plug giving the name AND colour of each wire

3)

- What will be the cost of using this number of units if 1 unit costs 7p

- The standing charge is £10.50. What is the total bill?

- What is economy 7?

- Who might use economy 7?

4) Using the formula for Power calculate the missing value for these appliances

| APPLIANCE | VOLTAGE | CURRENT | POWER |
| --- | --- | --- | --- |
| TAPE PLAYER | 12V | 2.5A |  |
| VIDEO RECORDER | 48V | 2.2A |  |
| ELECTRIC FIRE | 240V |  | 2800W |
| WALKMAN | 3V | 0.07A |  |
| SECURITY LIGHT | 240V |  | 60W |
| COMPUTER | 240V | 0.5A |  |
| MICROWAVE OVEN |  | 4A | 650W |

5)

| Area of house | %age of heat lost | Possible insulation | “Payback” time (years) |
| --- | --- | --- | --- |
| roof | 25% | Loft insulation | 2 |
| doors | 15% | draught proofing | 1 |
| windows | 10% | double glazing | 25 |
| walls | 35% | cavity wall insulation | 8 |

- For each type of insulation shown above say which type of heat transfer (CONDUCTION, CONVECTION, RADIATION) the insulation prevents.

- which type of insulation shown above is the cheapest?

- Which is the most cost effective way of insulating the house?

- Which is the least cost  effective way of insulating the house?

6) Calculate the efficiencies of the following power stations

| Type of power station | energy put in (MJ) | energy out of station(MJ) |
| --- | --- | --- |
| coal | 6250 | 2500 |
| oil | 5000 | 2250 |
| hydroelectric | 2693 | 1750 |
| nuclear | 4383 | 3200 |
| gas | 5278 | 2375 |
| wind | 1264 | 1075 |

- Which is the most efficient station

- Which of the power stations listed above use fossil fuels?

- Which of the power stations listed above use renewable energy sources

- For each of the stations say where some of the energy is lost

- Which type of energy source shown in the table best for building a power station? - give reasons

7) a) What is a fuse and what EXACTLY does it do

b) what fuse would an electrical toaster of  1080watts running off the mains (240V) need? (Show your calculations)-  hint  P= V  I

c) Copy out this diagram and show how this toaster might electrocute someone

d) How would the earth wire prevent electrocution?

8) A Wind farm has 35 turbines; each turbine produces 300 kW of electrical power

- How much electrical power does the site produce?

- How much electrical energy does the farm produce per hour? (use Power = Energy  time)

- Assuming 1 unit costs the consumer 7p. How much money will the farm make per hour?

- A coal fired power station produces 2500MW (2500,000,000W) how many more turbines will the farm need to match the coal power station

- Name 2 advantages of a wind farm over a coal fired power station

- What are the disadvantages of a wind farm?

- Describe the energy Transfers that take place in a turbine

9) Using your Knowledge of Particles or of The EM spectrum explain in detail how

- Conduction works

- Convection works

- Radiation works

- How heat escapes from a copper calorimeter